# STORM  WARNING  PROCESS 08

## Team Leaders:

### Tripper:

### Caleb

Loren

In preparation for 08 season we are conducting this meeting to update all staff as to what transpires once the decision has been made to “shut down operations”

It is important that each group understand the importance to each other in preparing the facility in the case of a Hurricane, and the expected cooperation as Legendary employees to the Business, when we prepare well, we can finish well and are able to reopen and operate in the shortest period following adverse conditions.

Each team leader has a responsibility to follow thru on each task and direct his crew to assist others where required.

Phone numbers:

In previous years we have used the “buddy” system, this year I would ask that each employee call his Team leader first, if not reachable, call the next on the list, it is imperative that we keep in contact during and post Hurricane.

Tripper 		xxx-xxxx

Loren		xxx-xxxx

Caleb			xxx-xxxx

Tripper’s Teams:

Yachts: Responsibilities

Yacht team are responsible to fuel, remove all canvas, anchor systems, re-locate vessel’s to Tucker, Shalimar Yacht Basin to be hauled OR anchored out.

Inspection of Ground Tackle/ loading and retrieval/ chaff guards. If vessel’s are anchored out, the clear intent is to be place on the East side of the Bridge in Rocky.

Securing of all vessel’s at MYC and Seaview/ DYC

Support vessel: fuel, check safety equipment/ oil, fenders and Operations

Anchors to be set at 30 degrees off transom.

Chase boat to retrieve “rode” and return to bow of vessel.

Chaff guards to be installed.

Two staff per vessel

Two bow anchors  per vessel

Keys to be left on board in cockpit

Turn off all Power

Remove all canvas where possible and stow on same vessel.

Install Anchor chaff guards.

SERVICE and PARTS

Loren:	Responsibilities

Entire Service/Parts and Rigging crew

Responsible to move all Service equipment to higher locations

Assist in moving all trailer vessels to A/B

EPA area

Removing awnings at Service entrance.

Remove and secure all Yacht canvas and loose equipment on E side.

Adding a total of 8 stands per units, adding line between stands/ pt to stb

Secure all Computers to higher ground

Install “rag” seals under all parts/ service doors

Removing all FG equipment from tents to A side [inside door]

Remove power/ air and water from Tents

Jack tents so wheels are free of alloy frame

Shut down Compressors and bleed off.

Check and make sure all drains and grids are clear ANY debris/

MARINA TEAM

CALEB: Responsibilities

Entire Marina team.

Lifting and storing all LMD/ FWB vessel’s in Barn

Securing Ship Store equipment

Removing Dock ramps

Shut down: fuel/ electrical

Moving Marina work shop equipment to higher ground

Fueling all Fork lifts, Hysters and Tractors

Lifting and storing all “non barn clients”

Tag each vessel arriving on transom.

C/D Door Post

Load C/D 1st

Load A/B 2nd

Place as many Sales units in racks/ TAG transoms as Sales.

Removing all hoses/ trash and clearing the deck.

Shut down all Water at Meters [2] in center Isle

Drop flags and store in SS

SALES SUPPORT TEAM: RESPONSIBILITIES

Responsible in moving all hard racks in support of Marina crew moving boats from Sales lot to Barn

Moving boats from FWB to either BB or Midbay

Securing all trash units in Sales areas

Boat stands, crib blocks

Moving Trailer shop equipment to work benches.

Secure Sales building outside equipment & awnings

Secure Sales “main door” PIN and lock

Fill all Company trucks and check engines

Hoses/ trash can [place inside Sales building]

Responsible for securing trailer shop/ Hoist hut and cables and entire Sales lot and Sales building.

Secure all trash facilities in barn

Secure Sales building/ Pin main Door. ( Add line to Hyster)

Sand bag bottom rack.

Fill ALL Company Trucks with fuel

72/48 Hour Notice

GO-NO GO

Team leader meet and go over present conditions, storm direction and force

All Yachts are checked for fuel and fuel added at that time.

Chase boat [Pursuit] brought down, fueled, fluids checked, safety gear checked and basic tools loaded [knife, flash lights, towing harness, life jackets for 6] on board

All Ground tackle is located and ready to move to hard deck ready to load on “ tender”/yachts.

48 Hour Notice

Team leader’s meet for final decision and direction.

GO – NO GO

24 hour Notice:

All Staff to complete assigned tasks

Secure entire property

Check all Yachts

Secure any loose materials in and around property

Yacht team to secure DYC and Seaview

Shut down all Computers

Property Shut down

12 Hour Notice:

LMD CLOSED

Returning to Legendary:

Please call or email your Leader within the 24 hours of Company closure for re-start schedule, if you are unable to contact leader, contact Tripper @ xxx-xxx-2525 [c]